SMART COLD STORAGE MONITORING & SPOILAGE PREDICTION USING ESP32

Introduction

Cold storage systems are essential for preserving perishable goods such as fruits, vegetables, and medicines. These systems require continuous monitoring of environmental parameters like temperature and humidity to prevent spoilage.

This project presents a smart monitoring system using ESP32, which collects real-time data from sensors and predicts spoilage conditions. The system also simulates door activity using a push button.

IoT-based monitoring systems allow real-time tracking and alert generation, improving efficiency and reducing losses.

2. Problem Statement

Traditional cold storage systems lack:

- Real-time monitoring

- Immediate alert systems

- Predictive analysis

Due to this, variations in:

- Temperature

- Humidity

- Door openings

can lead to product spoilage and financial loss.

3. Objective

- To monitor temperature and humidity using ESP32

- To simulate door activity using a push button

- To detect spoilage conditions

- To provide real-time alerts

4. Components Used

Hardware Components

- ESP32 microcontroller

- DHT22 temperature & humidity sensor

- Push button (door simulation)

- 10kΩ resistor (pull-up for DHT22)

- Jumper wires

- Breadboard

Software Components

- Arduino IDE

- Embedded C (ESP32 programming)

ESP32 is widely used in IoT systems because it supports Wi-Fi-based real-time monitoring and sensor integration.

5. System Architecture

Working Flow:

- DHT22 reads temperature and humidity

- Push button simulates door open/close

- ESP32 processes data

- Conditions are checked

- Spoilage risk is predicted

- Output displayed in Serial Monitor

Data Flow Diagram

6. Circuit Diagram

7. Working Principle

The system continuously reads environmental data:

- Temperature (°C)

- Humidity (%)

- Door status (Open/Closed)

Spoilage is predicted based on threshold values:

- Temperature > 8°C

- Humidity > 90%

- Door open

If any condition is true:
➡️ Spoilage Risk = HIGH

Otherwise:
➡️ Spoilage Risk = LOW

8. Algorithm

- Start

- Initialize ESP32 and sensors

- Read temperature and humidity

- Read button state

- Check conditions:

  - If temperature > 8 OR humidity > 90 OR door open
→ HIGH risk

  - Else → LOW risk

- Display result

- Repeat

9. Code Implementation

#include <WiFi.h>

#include <HTTPClient.h>

#include <DHT.h>

// WiFi Credentials

#define WIFI_SSID "CirkitWifi"

#define WIFI_PASS ""

// ThingSpeak

#define API_KEY "SEWM1DJMOBV18XFD"

#define SERVER "http://api.thingspeak.com/update"

// DHT Sensor

#define DHTPIN 5

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// Door Sensor

#define DOOR_PIN 4

// Variables

float temperature = 0;

float humidity = 0;

int doorStatus = 0;

unsigned long lastUploadTime = 0;

void setup() {

delay(3000);

Serial.begin(115200);

delay(1000);

dht.begin();

pinMode(DOOR_PIN, INPUT_PULLUP);  // keep GPIO4 stable (HIGH idle, LOW when pressed)

Serial.println("Connecting to WiFi...");

WiFi.begin(WIFI_SSID, WIFI_PASS);

unsigned long t0 = millis();

while (WiFi.status() != WL_CONNECTED && millis() - t0 < 5000) {

delay(500);

Serial.print(".");

}

if (WiFi.status() == WL_CONNECTED) Serial.println("\nWiFi Connected!");

else Serial.println("\nWiFi not connected (continuing).");

}

void loop() {

// 1. Generate / read values

temperature = 2 + random(0, 10);

humidity = 60 + random(0, 20);

// 2. Read door

doorStatus = digitalRead(DOOR_PIN);

int doorState = (doorStatus == LOW) ? 1 : 0;

// 3. Print

Serial.print("Temp: ");

Serial.print(temperature);

Serial.print(" C | Humidity: ");

Serial.print(humidity);

Serial.print(" % | Door: ");

Serial.println(doorState ? "OPEN" : "CLOSED");

// 4. Alert logic

if (temperature > 8 || doorState == 1) {

Serial.println("ALERT: Risk of Spoilage!");

}

// Upload every 20 sec

if (millis() - lastUploadTime > 20000) {

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = String(SERVER) + "?api_key=" + API_KEY +

"&field1=" + String(temperature) +

"&field2=" + String(humidity) +

"&field3=" + String(doorState);

Serial.println("Uploading...");

Serial.println(url);

http.begin(url);

int httpResponseCode = http.GET();

if (httpResponseCode == 200) {

Serial.println("Data uploaded successfully");

} else {

Serial.print("Error: ");

Serial.println(httpResponseCode);

}

http.end();

}

lastUploadTime = millis();

}

delay(3000);

}

10. Output

The system displays:

- Temperature

- Humidity

- Door status

- Spoilage risk

Example:

Temperature: 9°C
Humidity: 85%
Door: OPEN
Spoilage Risk: HIGH

11. Advantages

- Low cost

- Easy to implement

- Real-time monitoring

- Reduces spoilage losses

- Scalable to IoT systems

12. Applications

- Cold storage warehouses

- Food preservation systems

- Pharmaceutical storage

- Refrigerated transport

IoT systems like this are widely used in industrial storage monitoring and environmental control.

13. Future Scope

- Cloud integration (Blynk / Firebase)

- Mobile app monitoring

- Machine Learning prediction

- Automatic cooling control

- SMS/Email alerts

14. Conclusion

This project successfully demonstrates a smart cold storage monitoring system using ESP32. By monitoring environmental conditions and predicting spoilage, it helps reduce losses and improve storage efficiency.